⚙️ AutoModel Tab Plugin Tab 7 — Comprehensive Modeling Automation Suite Full Architecture & Feature Specification Version 1.0 ¦ March 2026 ¦ Based on Revit API 2024–2026

# EXECUTIVE SUMMARY

The AutoModel Tab is the 7th dedicated ribbon tab in the plugin suite, purpose-built to eliminate the most time-consuming and repetitive modeling tasks in Autodesk Revit. Drawing from deep research into the Revit API, existing market tools, and emerging AI workflows, this tab consolidates seven powerful automation panels — each targeting a different source type or modeling challenge — into one cohesive, workflow-driven experience.

CORE MISSION: Convert any input source — DWG, PDF, image, point cloud, Excel, or AI prompt — into an accurate, parametric Revit BIM model with minimal manual intervention.

Market research confirms the demand for this toolset:

- WiseBIM AI (Revit App Store, 2024) proved that AI-driven 2D-to-3D conversion achieves 70–90% accuracy and speeds up workflows by 20x.

- Kingspan Isoeste reduced a 2-day DWG-to-Revit workflow to 1 hour with a custom CAD layer-based automation plugin (DiRoots case study).

- Undet for Revit demonstrated that Scan-to-BIM point cloud modeling can be nearly halved in time with intelligent assist tools.

- The Revit API (2024–2025) provides deep access to NewFamilyInstance(), NewWalls(), FamilyManager, FilteredElementCollector, and batch creation methods — all exploitable for automation at scale.

# SECTION 1: MARKET LANDSCAPE & GAP ANALYSIS

## 1.1 Existing Tools — What's Already Out There

| Tool | Category | Capability | Limitation / Gap |
| --- | --- | --- | --- |
| WiseBIM AI | Image/PDF/DWG → Revit | Walls, doors, windows, slabs via AI detection | No layer control, wall types wrong, family duplicates |
| Undet for Revit | Point Cloud Assist | Fit Wall/Column from 3 cloud points; auto floors/ceilings | Requires RCP format; no AI element detection |
| DiRoots Custom | DWG Layer → BIM | Layer-to-element mapping; auto curtain walls, doors, ceilings | Custom per project; no universal layer-mapping UI |
| BIMDeX | AutoCAD → Revit | Converts 3D AutoCAD blocks to Revit families via BXF format | Requires AutoCAD + Revit both installed; US-only |
| Plans2BIM | Online AI Service | PDF/JPEG → 3D BIM (cloud-based) | Not Revit-native; external upload; privacy concerns |
| IMAGINiT Scan2BIM | Point Cloud → MEP | Auto extraction of pipes, ducts from scan data | MEP-focused; expensive; limited architectural support |
| pyRevit | Scripting Platform | NewFamilyInstance placement via Python scripting | Requires scripting knowledge; no GUI family tools |
| Room Finishing | Auto Element Gen | Auto generates skirting boards, floor finishes by room | Limited to room finishes only |

## 1.2 Identified Market Gaps — Our Opportunity

- No single tool covers ALL input types: DWG, PDF, image, point cloud, Excel, and AI prompts.

- No tool offers intelligent layer-to-family mapping with a user-configurable UI.

- No tool handles batch family creation from parametric data (e.g., Excel sheets defining thousands of family instances).

- No tool integrates in-place family generation with reference plane automation inside the Revit Family Editor workflow.

- No tool provides a DWG-to-BIM pipeline with wall-type matching, constraint attachment, and hosted element auto-placement.

- AI tools (WiseBIM) suffer from family duplication, misaligned walls, and no post-processing cleanup — a solved problem with proper API usage.

# SECTION 2: REVIT API OPPORTUNITIES EXPLOITED

## 2.1 Core Creation APIs

These are the foundational Revit API methods that power every AutoModel tool panel.

| API Class / Method | Function | AutoModel Use Case |
| --- | --- | --- |
| Document.Create.NewFamilyInstance() | Places any loadable family symbol at XYZ, on face, on wall, on level | Auto-placing doors, windows, furniture, MEP from any data source |
| Document.Create.NewFamilyInstance() (batch via NewFamilyInstances2) | Batch creates multiple family instances in one transaction | Excel/CSV bulk placement of 100s of columns or fixtures |
| Document.Create.NewWall() | Creates walls from curves with type, level, height, structural flag | DWG polyline → Revit wall; image line detection → wall |
| Document.Create.NewFloor() | Creates floor elements from profile curves | Point cloud auto floor; room-boundary floor generation |
| FamilyManager.AddParameter() / SetFormula() | Programmatically add/set family parameters in RFA context | AI Family Creator: auto-build parametric families from specs |
| Document.LoadFamily() + FamilySymbol.Activate() | Loads an RFA file and activates a type for placement | Batch family loader before mass placement operations |
| FilteredElementCollector + OfCategory() | Queries all elements by category, class, or parameter filter | Detect walls/floors for hosted element placement |
| ImportInstance (DWG link) + GetGeometryObjectFromReference() | Accesses geometry of linked CAD files by layer | DWG layer-to-wall: extracts curves per layer for wall creation |
| PointCloudInstance + PointCollection | Reads XYZ data from linked RCP point clouds | Scan-to-model: fit walls/slabs from point cloud planes |
| HostObjectUtils.GetSideFaces() + NewFamilyInstance(Reference, ...) | Creates face-hosted family instances on wall/floor faces | Auto-place lighting, electrical on room faces post-wall creation |
| Design Automation for Revit (APS) API | Cloud-based headless Revit for family creation and model generation | Server-side AI family builder; external model generation service |

## 2.2 Advanced API Techniques

### Transaction Management

- Use SubTransaction groups for each element type (walls, then floors, then families) to allow granular rollback if one category fails.

- Use TransactionGroup to batch all AutoModel operations under one Undo action for the user.

### Geometry Extraction from CAD

- GeometryElement.GetEnumerator() → GeometryObject → Solid/Curve extraction from ImportInstance nodes.

- Layer filtering via ImportInstance.GetLayerNames() then GeometryFilter by layer name.

### Parameter Automation

- FamilyManager.CurrentType / AddParameter / SetValueString for building families programmatically.

- ParameterFilterElement for automated view filter generation after bulk placement.

### In-Memory Family Creation

- Create an RFA Document in memory via Application.NewFamilyDocument(template), add extrusions, reference planes, parameters, then save and load — all without ever opening the Family Editor UI.

# SECTION 3: THE 7 PANELS — AUTOMODEL TAB ARCHITECTURE

The AutoModel Tab ribbon is organized into 7 functional panels, each addressing a distinct workflow:

| # | Panel Name | Core Function | Primary Inputs |
| --- | --- | --- | --- |
| P1 | CAD → BIM | Convert DWG/DXF layers to Revit walls, floors, roofs, columns | Linked DWG/DXF file |
| P2 | Image/PDF → BIM | AI-detect building elements from scanned plans/images | PDF, JPEG, PNG, TIFF file |
| P3 | Scan → BIM | Semi-auto and AI-assisted point cloud to Revit model | Linked RCP/RCS point cloud |
| P4 | Family Creator | Build parametric Revit families from specs without opening Family Editor | Parameters, geometry input, AI prompt |
| P5 | Family Placer | Intelligent bulk family placement from data sources | Excel/CSV, CAD blocks, selection rules |
| P6 | Room → Model | Auto-generate walls, floors, ceilings, finishes from room data | Revit rooms/spaces, Excel room schedules |
| P7 | AI Modeler | Natural language + Claude AI to generate Revit model elements | Text prompt, reference elements |

## PANEL 1 — CAD → BIM  (DWG/DXF to Revit Elements)

Inspired by: DiRoots Kingspan case study (2-day workflow → 1 hour). Exploits: ImportInstance API, NewWall(), NewFloor(), FilteredElementCollector.

### Overview

This panel reads a linked CAD file and maps its layers to Revit element creation commands. Users configure a layer-to-type mapping table (saved as a project template) and the system creates parametric Revit elements from the CAD geometry.

### Tools in Panel 1

| CAD LINK MANAGER | Manages linked DWG/DXF files; shows layer list with element count per layer. Toggle layer visibility, merge layers, filter by color or line weight. |
| --- | --- |

| LAYER MAPPER | Visual spreadsheet UI: map each CAD layer to a Revit element type (Wall, Floor, Column, Ceiling, Roof, Beam, ModelLine, etc.). Save and load mapping templates (JSON). Includes auto-suggest: matches layer names like "WALL", "A-WALL-EXTR" automatically. |
| --- | --- |

| WALL BUILDER | Extracts polylines/lines from selected layers and creates Revit walls. Options: wall type selector, base level, top constraint, offset, structural/non-structural toggle. Handles open endpoints and T-junctions with join geometry. |
| --- | --- |

| FLOOR / CEILING BUILDER | Detects closed boundary polygons from CAD layers and creates Revit floors or ceilings. Auto-applies room boundary flag. Height offset control. |
| --- | --- |

| COLUMN / BEAM PLACER | Reads CAD block insertion points or lines from structural layers and places Revit structural columns or beams. Handles rotation from CAD block angle. |
| --- | --- |

| OPENING CUTTER | Detects door/window CAD symbols by layer and places Revit hosted door/window families at corresponding wall locations. Family auto-match by symbol size. |
| --- | --- |

| DWG CLEANUP POST-MODEL | After modeling, hides or detaches the CAD link. Optionally joins intersecting walls. Removes short wall fragments (below user-defined minimum length). Aligns walls to nearest grid line. |
| --- | --- |

### Revit API Methods Used (Panel 1)

- ImportInstance.GetLayerNames() — list all layers in linked CAD

- ImportInstance.GetGeometryObjectFromReference() — extract curves per layer

- Document.Create.NewWall(curve, wallType, level, height, offset, flip, structural)

- Document.Create.NewFloor(profile, floorType, level, structural)

- Document.Create.NewFamilyInstance(location, symbol, host, StructuralType.NonStructural)

- JoinGeometryUtils.JoinGeometry() — wall joining post-creation

## PANEL 2 — Image/PDF → BIM  (AI Plan Interpreter)

Inspired by: WiseBIM AI (Autodesk App Store 2024). Exploits: Computer Vision API (CV backend) + Revit API creation methods. 20x speed improvement proven in market.

### Overview

This panel ingests a floor plan image (PDF, JPEG, PNG, TIFF, scanned blueprint) and uses AI computer vision to detect walls, openings, rooms, and structural elements — then generates corresponding Revit model elements. Unlike WiseBIM, our implementation provides post-processing correction tools and respects existing project family types.

### Tools in Panel 2

| IMAGE IMPORTER | Import PDF (page selector), JPEG/PNG/TIFF. Auto-scale calibration: user draws a reference dimension on the image to set scale. DPI/resolution display. Supports multi-page PDF floor plans (one level per page). |
| --- | --- |

| AI ELEMENT DETECTOR | Runs computer vision analysis on the uploaded plan. Detects: exterior walls, interior walls, doors (with swing direction), windows, stairs, elevators, structural columns. Displays detected elements overlaid in color on the preview. Confidence score per element. User can accept/reject each detected category. |
| --- | --- |

| WALL TYPE MATCHER | Maps detected wall thickness values to existing Revit wall types. User-configurable matching table. Tolerance slider (e.g., ±10mm). Option to create new wall types if no match found, or unify all to a single type. |
| --- | --- |

| LEVEL ASSIGNER | Assigns detected elements to a Revit level. Multi-level plans (stacked floors in one image) supported via zone-selection or page-by-level assignment. |
| --- | --- |

| BIM GENERATOR | One-click generation from reviewed detections. Creates: walls (NewWall), floors (NewFloor), door/window families (NewFamilyInstance with host), and room separation lines. Progress bar with element-type counters. |
| --- | --- |

| POST-GENERATION FIXER | Detects common AI errors post-generation: duplicate overlapping walls, walls not joined at corners, orphaned door/window instances. Auto-fix or manual selection for each issue type. Wall alignment snap to nearest grid/reference plane. |
| --- | --- |

### Revit API Methods Used (Panel 2)

- Document.Create.NewWall() — main wall creation from detected line segments

- Document.Create.NewFamilyInstance(curve/point, symbol, wall, level, StructuralType.NonStructural) — hosted doors/windows

- WallUtils.DisallowWallJoinAtEnd() / AllowWallJoinAtEnd() — join control

- Document.Create.NewRoomBoundaryLines() — room separation lines from detected room boundaries

- FilteredElementCollector.OfClass(typeof(WallType)) — retrieve existing wall types for matching

## PANEL 3 — Scan → BIM  (Point Cloud to Revit)

Inspired by: Undet for Revit (Fit Wall from 3 points), IMAGINiT Scan2BIM, CloudWorx. Exploits: PointCloudInstance API, plane-fitting algorithms, NewWall/NewFloor.

### Overview

This panel bridges laser scan data (linked RCP/RCS point clouds) with Revit modeling. It provides both manual-assist tools (like Undet's 3-point wall fit) and automated AI element detection from point cloud planes. Targets renovation, heritage, and as-built projects.

### Tools in Panel 3

| CLOUD NAVIGATOR | Slice viewer: create horizontal/vertical point cloud slices without leaving Revit. Control slice thickness, position, and density. Navigate through floor levels using slider control. Much lighter than importing full RCP into all views. |
| --- | --- |

| FIT WALL (3-POINT) | User clicks 3 points in a cloud slice → system fits a wall line, detects wall thickness from cloud density, creates a Revit wall. Options: single wall or chain-mode (continuous wall path). Wall type auto-suggested by detected thickness. |
| --- | --- |

| FIT COLUMN (2-POINT) | User clicks 2 diagonal points of a column cross-section in cloud → system creates a structural column of matching size. Rotation auto-detected from cloud point distribution. |
| --- | --- |

| AUTO FLOOR / CEILING | Detects dominant horizontal planes in a cloud level. Automatically creates Revit floors/ceilings at detected elevations. Threshold control to ignore noise/pipes. Multiple planes per level supported. |
| --- | --- |

| OPENING DETECTOR | Analyzes wall segments for density gaps (doors, windows). Places Revit door/window families at gap locations. Width/height detected from cloud extent. User reviews proposed openings before placement. |
| --- | --- |

| DEVIATION CHECKER | After modeling, compares Revit elements against the point cloud. Color-heatmap overlay showing deviation (green = within tolerance, red = off). Sets modeled element "Modeling Tolerance" parameter. Export deviation report to Excel. |
| --- | --- |

| FAMILY FROM SCAN | Advanced: user selects a cloud region → system extracts the geometry → creates a Revit Generic Model in-place family matching the scan shape. Useful for custom architectural elements and equipment. Simplification level control (LOD slider). |
| --- | --- |

### Revit API Methods Used (Panel 3)

- PointCloudInstance.GetPoints(PointCloudFilter, density) — extract cloud points in defined region

- Plane.CreateByThreePoints() — fit reference planes from cloud point sets

- Document.Create.NewWall(line, wallTypeId, levelId, height, ...) — wall from fitted line

- Application.NewFamilyDocument(genericModelTemplate) — create in-memory family for complex shapes

- FamilyItemFactory.NewExtrusion() — geometry creation inside family document from cloud outline

## PANEL 4 — Family Creator  (Parametric Family Builder)

Inspired by: APS Design Automation family creation sample, FamilyManager API, BIMDeX block conversion. Exploits: NewFamilyDocument(), FamilyManager, in-memory RFA creation.

### Overview

This panel allows users to create parametric Revit families (RFA files) programmatically — without ever manually opening the Family Editor. Define geometry, parameters, and connectors through a structured UI or AI prompt, and the plugin generates the complete RFA.

### Tools in Panel 4

| TEMPLATE SELECTOR | Choose from all Revit family templates: Generic Model, Door, Window, Furniture, Structural Column, Pipe Fitting, etc. Shows template preview and available reference planes. Option to start from an existing RFA as a base. |
| --- | --- |

| GEOMETRY BUILDER | Visual form builder for common shapes: box extrusion, cylinder, revolve, sweep path. Set dimensions as formula-linked parameters. Layer geometry forms together. For complex shapes: import from DWG 3D block and convert to family extrusion automatically. |
| --- | --- |

| PARAMETER MANAGER | Add, rename, group, and set formulas for family parameters. Type vs Instance toggle. Shared parameter file integration. Bulk import parameters from Excel. Auto-creates standard LOD parameters. |
| --- | --- |

| CONNECTOR WIZARD | For MEP families: add pipe/duct/electrical connectors with auto-size and direction. Structural connection type setter. Opening cut automation for hosted families. |
| --- | --- |

| TYPE CATALOG BUILDER | Define multiple family types in a spreadsheet-style grid within the UI. Each column is a parameter; each row is a type. Export as .txt Type Catalog. Import existing type catalogs from Excel. |
| --- | --- |

| BATCH FAMILY BUILDER | Generate multiple family files from an Excel specification sheet. Each row = one family variant. Output folder management. Version control with timestamp suffix. |
| --- | --- |

| AI FAMILY ASSISTANT | Text prompt: "Create a rectangular ceiling light fixture, 600x600mm, with 4 types by lumen output, includes electrical connector." — AI interprets and pre-fills all builder panels. User reviews and generates. |
| --- | --- |

### Revit API Methods Used (Panel 4)

- Application.NewFamilyDocument(familyTemplatePath) — create RFA in memory

- FamilyManager.AddParameter(name, ForgeTypeId groupId, ParameterType, isInstance)

- FamilyManager.SetFormula(FamilyParameter, formulaString)

- FamilyItemFactory.NewExtrusion(profile, refPlane, start, end)

- FamilyItemFactory.NewRevolution(), NewSweep(), NewBlend() — other geometry types

- FamilyManager.MakeType(typeName) — add new types to family

- Document.SaveAs(path) — save RFA; Document.LoadFamily() — reload into project

## PANEL 5 — Family Placer  (Intelligent Bulk Placement)

Exploits: NewFamilyInstance (all overloads), NewFamilyInstances2 (batch), FilteredElementCollector, HostObjectUtils. Proven by: batch column placement SDK samples.

### Overview

The most powerful bulk-placement engine for Revit families. Supports placement from Excel/CSV coordinates, CAD block positions, Revit room centroids, grid intersections, wall-hosted placement rules, and face-hosted rules. All placements happen in a single batched transaction for maximum performance.

### Tools in Panel 5

| EXCEL PLACER | Import Excel/CSV with columns: FamilyName, TypeName, X, Y, Z, Level, Rotation, and any parameter overrides. Preview placement in 3D before committing. Field mapping UI for non-standard column names. Supports 10,000+ instances efficiently via batch API. |
| --- | --- |

| GRID PLACER | Place families at all intersections of selected Revit grids. Offset X/Y from intersection center. Optional: place only at specific intersection labels (e.g., A1 through D6). Type selector. Structural column batch creation optimized. |
| --- | --- |

| ROOM PLACER | Place families at room centroids or custom offsets within rooms. Filter rooms by name, department, number pattern. Place multiple families per room (e.g., smoke detector + sprinkler + light fixture). Quantity per area option (e.g., 1 per 9m²). |
| --- | --- |

| CAD BLOCK PLACER | Reads insertion points of CAD blocks from a linked DWG and places corresponding Revit families at those locations. Block-name to family-type mapping table. Rotation and scale transfer. Handles nested blocks. |
| --- | --- |

| WALL FACE PLACER | Places face-hosted families (switches, outlets, fixtures) on interior/exterior wall faces. Rule-based: place at 300mm above floor; every X meters along wall; at door center ±300mm. Uses HostObjectUtils.GetSideFaces() + NewFamilyInstance(Reference...). |
| --- | --- |

| PATTERN PLACER | Place families in geometric patterns: linear array, radial array, grid array, random scatter (within bounded region). Parametric spacing controls. Good for lighting layouts, structural arrays, site furniture. |
| --- | --- |

| SMART REPLACE | Replace existing placed families with a new type while preserving location, rotation, host, and shared parameter values. Batch replace across entire model or selection. Family-to-family parameter mapping for data transfer. |
| --- | --- |

### Revit API Methods Used (Panel 5)

- Document.Create.NewFamilyInstances2(IList<FamilyInstanceCreationData>) — batch creation, all instances in one call

- FamilyInstanceCreationData(XYZ, FamilySymbol, Level, StructuralType) — data container per instance

- Document.Create.NewFamilyInstance(Reference face, XYZ, XYZ dir, FamilySymbol) — wall face placement

- HostObjectUtils.GetSideFaces(wall, ShellLayerType.Interior) — get wall faces for face-hosted placement

- Grid.GetCurvesInView() / Grid.Curve — extract grid line geometry for intersection calculation

- Room.GetBoundarySegments() — room boundary for containment checks

## PANEL 6 — Room → Model  (Room-Driven Auto Modeling)

Inspired by: Room Finishing plugin (Revit App Store). Extended to full room-driven BIM generation including walls, ceilings, floors, furniture, MEP rough-ins.

### Overview

This panel takes Revit rooms (or spaces) as the driver for automated modeling. From a room schedule or room selection, it generates all associated model elements: partition walls, floor finishes, ceiling types, skirting/base boards, and MEP rough-in placeholders.

### Tools in Panel 6

| ROOM SCHEDULE IMPORTER | Import room data from Excel: Room Number, Name, Type, Area, Height, Wall Finish, Floor Finish, Ceiling Finish, MEP requirements. Match to existing Revit rooms by number. Supports room creation if rooms don't exist yet. |
| --- | --- |

| FLOOR FINISH GENERATOR | For selected rooms: create floor finish elements using room boundary as profile. Assign floor type by room type mapping. Handles multi-material rooms (e.g., bathroom with tile border and mosaic center) via sub-regions. |
| --- | --- |

| CEILING PLACER | Auto-creates ceiling elements within room boundaries at specified height. Ceiling type mapping by room type. Handles ceiling offsets from level. Slope ceilings for drainage rooms. |
| --- | --- |

| BASE/SKIRTING GENERATOR | Places wall-sweep or detail-component skirtings around room perimeter at floor level. Configurable height and profile family. Miter at corners. Gap at door openings (auto-detected from hosted doors). |
| --- | --- |

| PARTITION WALL BUILDER | From a room layout plan (imported or defined in panel), auto-generates interior partition walls separating room groups. Wall type by room boundary rule. Checks for existing walls to avoid duplicates. |
| --- | --- |

| MEP ROUGH-IN PLACER | Places MEP placeholder families in rooms based on room type rules (e.g., bathroom → floor drain + 2x wall outlets + 1x ceiling light). Integrates with panel 5 Room Placer. Generates MEP coordination list for engineers. |
| --- | --- |

## PANEL 7 — AI Modeler  (Natural Language BIM Generation)

FUTURE-FORWARD: Uses Claude API (or similar LLM) to interpret architectural descriptions and generate Revit model elements via structured API commands. Unique in the market.

### Overview

The AI Modeler is the most innovative panel, allowing users to describe what they want in natural language and have the plugin generate the corresponding Revit model elements. The AI understands spatial relationships, building typologies, and Revit family names.

### Example Prompts

| EXAMPLE 1 | "Create a 10m x 8m rectangular room with 200mm exterior walls, a central corridor wall, 3 windows on the south face, and a flat ceiling at 2.8m." |
| --- | --- |

| EXAMPLE 2 | "Place 6 structural columns in a 5m grid starting from grid intersection A1, all concrete 400x400, from Level 1 to Level 3." |
| --- | --- |

| EXAMPLE 3 | "Replicate this room's layout pattern (selection) 4 times along grid line A, spacing 7.5m, mirrored on alternate bays." |
| --- | --- |

### Tools in Panel 7

| PROMPT CONSOLE | Natural language input with context: current view, selected elements, and active level are automatically provided to the AI as context. Response preview shows planned actions before execution. Undo history integration. |
| --- | --- |

| ELEMENT DESCRIBER | User selects an existing element → AI describes it in text → user can say "Make 10 more like this" or "Create a mirrored version." Enables replication without family knowledge. |
| --- | --- |

| SCRIPT GENERATOR | AI generates pyRevit/Dynamo scripts from natural language. Saves to project as a reusable automation macro. "Next time someone says [trigger phrase], run this." Bridges AI and scripting workflows. |
| --- | --- |

| LAYOUT OPTIMIZER | AI suggests optimal family placement layouts based on constraints: "Maximize daylight in this room layout" or "Place columns to minimize beam spans." Integrates with Panel 5 for execution. |
| --- | --- |

# SECTION 4: TECHNICAL ARCHITECTURE

## 4.1 Plugin Structure

| Component | Description |
| --- | --- |
| AutoModel.Ribbon | Ribbon tab definition, button icons, panel layout. Uses Autodesk.Revit.UI.RibbonPanel + PushButton. |
| AutoModel.Core | Shared utilities: transaction managers, geometry helpers, unit converters, element finders. |
| AutoModel.CAD | Panel 1 logic: DWG parsing, layer extraction, curve → wall/floor conversion engine. |
| AutoModel.Vision | Panel 2 logic: image preprocessing, AI detection API client, coordinate transformation, element creation. |
| AutoModel.Scan | Panel 3 logic: point cloud slice engine, plane fitting, gap detection, deviation calculation. |
| AutoModel.Family | Panel 4 logic: in-memory RFA creation, parameter management, type catalog, AI family interpreter. |
| AutoModel.Placer | Panel 5 logic: bulk placement engine, Excel parser, grid/room calculators, face placement. |
| AutoModel.Room | Panel 6 logic: room-driven element generation, finish/MEP rule engine, schedule importer. |
| AutoModel.AI | Panel 7 logic: LLM client (Claude API), prompt context builder, action interpreter, script generator. |
| AutoModel.UI | Shared WPF UI components: data grids, preview canvases, progress dialogs, mapping tables. |
| AutoModel.Settings | Per-project and global settings storage using Revit extensible storage (DataStorage element). |

## 4.2 Data Flow Architecture

INPUT → PARSER → MAPPER → VALIDATOR → REVIT API → MODEL

- INPUT: File (DWG/PDF/Image/Excel), Point Cloud, User Selection, AI Prompt

- PARSER: Extracts raw geometry or data (curves, points, text, coordinates)

- MAPPER: Converts raw data to Revit creation parameters using user-defined or AI-inferred mapping

- VALIDATOR: Checks for conflicts, duplicates, out-of-tolerance data; returns warnings

- REVIT API: Executes creation within a TransactionGroup (single Undo)

- MODEL: Generated elements tagged with AutoModel_Source parameter for tracking

## 4.3 Settings & Configuration System

- Project-level mapping templates saved as JSON in Revit ExtensibleStorage (persists in RVT file)

- Global user preferences in %AppData%\AutoModel\settings.json

- Mapping templates exportable/importable as .ammap files for sharing across projects

- All AutoModel-created elements tagged with a shared parameter "AutoModel_GeneratedBy" (Panel name + timestamp)

## 4.4 Performance Considerations

- All bulk creations use Document.Create.NewFamilyInstances2() (batch API) — 10–50x faster than individual NewFamilyInstance() calls in a loop.

- TransactionGroup + SubTransactions per category allow partial rollback without failing entire operation.

- Point cloud operations use spatial indexing (octree) to limit query regions and avoid iterating all cloud points.

- Image/PDF processing runs in a background thread; Revit API calls execute on the main thread via ExternalEvent.

- AI API calls (Panel 7) are async with progress indication; Revit model operations execute only after full AI response received.

# SECTION 5: UI/UX DESIGN SPECIFICATION

## 5.1 Ribbon Tab Layout

AutoModel Tab → 7 Panels, left-to-right workflow order. Each panel has 1 large launch button + 2-4 small utility buttons below.

| 🏗 CAD→BIM | 🖼 IMG→BIM | ☁ SCAN→BIM | ⚙ FAM.CREATE | 📍 FAM.PLACE | 🏠 ROOM→MDL | 🤖 AI MODEL |
| --- | --- | --- | --- | --- | --- | --- |

## 5.2 Common UI Patterns

- Every tool follows a 3-step wizard: (1) Configure, (2) Preview, (3) Execute.

- Preview pane: shows what will be created as a 2D plan overlay before committing to Revit.

- All operations show a real-time progress bar with element-type counts.

- "Save Config" button on every tool saves settings as a named preset.

- "Dry Run" mode: creates no elements, only validates inputs and shows a report.

- Post-run summary panel: "X walls created, Y doors placed, Z warnings" with click-to-select links.

## 5.3 Accessibility & UX

- All fields have tooltips referencing Revit API docs and example values.

- Color-blind friendly status colors (icons + labels, not color alone).

- Keyboard shortcut: AM+[1-7] opens the corresponding panel (configurable).

- Localization-ready: all strings in resource files for multi-language support.

# SECTION 6: DEVELOPMENT ROADMAP

| Phase | Timeline | Deliverables | Priority |
| --- | --- | --- | --- |
| Phase 1 | Months 1–3 | Panel 5 (Family Placer: Excel + Grid + Room), Panel 1 basic (Layer Mapper + Wall Builder) | 🔴 Critical — highest ROI |
| Phase 2 | Months 4–6 | Panel 1 complete (all elements), Panel 6 (Room → Model: floors, ceilings, skirtings) | 🟠 High |
| Phase 3 | Months 7–9 | Panel 3 (Scan → BIM: cloud navigator, fit wall, auto floor, deviation), Panel 4 basic (Family Creator: geometry builder + param manager) | 🟠 High |
| Phase 4 | Months 10–12 | Panel 2 (Image/PDF → BIM: AI detection + BIM generator), Panel 4 advanced (batch builder, type catalog, AI assistant) | 🟡 Medium |
| Phase 5 | Months 13–18 | Panel 7 (AI Modeler: full natural language BIM generation, script generator, layout optimizer) | 🟢 Innovation |

## 6.1 Testing Strategy

- Unit tests: mock Revit API objects; test geometry math, layer parsing, coordinate transforms independently.

- Integration tests: test model (standard Revit sample project) with scripted test cases for each panel.

- Performance benchmarks: measure time for 1,000 family placements (target: <5 seconds on standard hardware).

- User acceptance testing: 3 pilot firms (architectural, structural, MEP) — 2 weeks each on real projects.

# SECTION 7: COMPETITIVE ADVANTAGE SUMMARY

| Feature | AutoModel Tab | Best Competitor |
| --- | --- | --- |
| All input types in one plugin | ✅ DWG + PDF + Image + Cloud + Excel + AI | ❌ Each tool covers 1–2 inputs only |
| Family type preservation | ✅ Matches to existing project types; no duplicates | ❌ WiseBIM creates duplicate family types |
| Batch placement API | ✅ NewFamilyInstances2 for 50x speed | ❌ Most tools loop individual NewFamilyInstance() |
| In-memory family creation | ✅ No Family Editor UI needed | ❌ No competitor offers this in a UI |
| Post-generation cleanup | ✅ Built-in fixer for AI errors | ❌ Users must manually fix AI output |
| Project settings persistence | ✅ Mapping templates in RVT via ExtensibleStorage | ❌ Most tools reset every session |
| Natural language modeling | ✅ Panel 7 AI Modeler (unique) | ❌ No competitor offers this |
| Deviation / QA reporting | ✅ Built into Scan panel | ⚠️ Only in separate expensive tools (Verity/ClearEdge) |

The AutoModel Tab is positioned as the definitive all-in-one modeling automation solution for Revit — the only tab that takes you from any data source to a clean, parametric BIM model with maximum API performance and minimum manual work.

— End of AutoModel Tab Plan Document —